AQA GCSE PHYSICS  ·  PAPER 1  ·  FOUNDATION & HIGHER

Electrical power

Electricity   ·   Lesson 6 of 10

Warm-up

Answer each one, then check.

1.   What is the unit of power?

Watt (W)

2.   Write the equation linking energy and time for power.

P = E ÷ t

3.   What is V = IR used for?

Linking pd, current and resistance

4.   Convert 2.3 kW to watts.

2300 W

5.   Work out 3².

9

Learning Objectives

1.   Explain how the power transfer in a device is related to the pd across it and the current through it.

2.   Recall and apply P = VI.

3.   Recall and apply P = I²R.

4.   Rearrange to find current, pd or resistance.

Electrical Power

power = potential difference × current      P = VI

Also P = I²R (power = current squared × resistance). Power is in watts (W), pd in volts (V), current in amperes (A) and resistance in ohms (Ω).

Power in a Circuit

A device with a bigger pd or current has a greater power.

A lamp with a voltmeter and ammeter and the power calculated as pd times current.

Using P = VI

A kettle has a current of 10 A when connected to the 230 V mains. Calculate its power.

1.   Write the equation

P = VI

2.   Substitute

P = 230 × 10

3.   Answer

P = 2300 W

Answer:   2300 W (2.3 kW)

Using P = I²R

A current of 6.0 A flows through a 5.0 Ω resistor. Calculate the power.

1.   Write the equation

P = I²R

2.   Square the current first

6.0² = 36

3.   Substitute

P = 36 × 5.0

4.   Answer

P = 180 W

Answer:   180 W

Finding the Current

A 60 W lamp is connected to the 230 V mains. Calculate the current.

1.   Rearrange

I = P ÷ V

2.   Substitute

I = 60 ÷ 230

3.   Answer

I = 0.26 A

Answer:   0.26 A

Which Equation?

Look at what you are given.

| You have | Use |
| --- | --- |
| V and I | P = VI |
| I and R | P = I²R |
| V and R | Find I first with I = V ÷ R, then P = VI |
| E and t | P = E ÷ t |

Common Mistakes

Take care with squares.

▸   Squaring.   Square only the current, then multiply by the resistance.

▸   Units.   Convert mA to A and kW to W first.

▸   Rating labels.   The power rating of an appliance is at its normal pd.

Key Terms

| Power The rate at which energy is transferred. | Watt One joule per second. |
| --- | --- |
| Power rating The power an appliance transfers in normal use. | Kilowatt 1000 watts. |
| Potential difference Energy transferred per coulomb of charge. | Current The rate of flow of charge. |

Your Task: Rating Plate

10 minutes

A hair dryer is rated 1500 W at 230 V. Calculate the current. A toaster takes 4.0 A at 230 V; find its power. 1. Rearrange P = VI for current. 2. Then calculate the toaster power.

A good answer shows:   Hair dryer: 1500 ÷ 230 = 6.5 A. Toaster: 230 × 4.0 = 920 W.

Can I...?

☐   Recall P = VI.

☐   Recall P = I²R.

☐   Choose the right equation.

☐   Rearrange for current.

☐   Square the current correctly.

☐   Convert units to W and A.

☐   Explain what power means.

☐   Give the unit.

Summary

✓   P = VI.

✓   P = I²R.

✓   Power is in watts.

✓   Higher power means faster energy transfer.

EXAM FOCUS A kettle draws 10 A from the 230 V mains. Calculate its power. (2 marks) Choose P = VI, substitute and give W.