AQA GCSE PHYSICS  ·  PAPER 1  ·  FOUNDATION & HIGHER

Current, resistance and potential difference

Electricity   ·   Lesson 2 of 10

Warm-up

Answer each one, then check.

1.   Write the unit of resistance.

Ohm (Ω)

2.   What is the unit of potential difference?

Volt (V)

3.   Where is a voltmeter connected?

In parallel across the component

4.   What does a longer wire do to resistance?

Increases it

5.   What is a current?

A flow of charge

Learning Objectives

1.   Explain that current depends on resistance and potential difference.

2.   Recall and apply V = IR.

3.   Describe how to investigate the resistance of a wire and of combinations of resistors (Required Practical 3).

4.   Interpret graphs of resistance against length.

V = Ir

potential difference = current × resistance      V = IR

The greater the resistance of a component, the smaller the current for a given potential difference across it. Questions use the term potential difference; voltage also gains credit.

Required Practical 3: Resistance of a Wire

Change only the length of wire between the clips.

A circuit with a cell, switch and ammeter in series with a wire on a metre rule, and a voltmeter across the length of wire being tested.

Method: Resistance and Length of a Wire

Required Practical 3, part 1.

| 1 Set up Connect the circuit with the ammeter in series and the voltmeter across the wire. | 2 Set the length Clip the crocodile clips at 0.20 m, and record the pd and current. | 3 Switch off between readings This keeps the wire at a constant temperature. | 4 Repeat Take readings at 0.40 m, 0.60 m, 0.80 m and 1.00 m. | 5 Calculate For each length R = V ÷ I. | 6 Plot Draw a graph of resistance against length: a straight line through the origin. |
| --- | --- | --- | --- | --- | --- |

Method: Resistors in Series and Parallel

Required Practical 3, part 2.

| 1 Series Connect known resistors in series, measure V and I, calculate R = V ÷ I. | 2 Parallel Repeat with the same resistors in parallel. | 3 Compare The series combination has a larger resistance than either resistor; the parallel combination has a smaller one. |
| --- | --- | --- |

Calculating Current

A 12 V battery is connected across a 4.0 Ω resistor. Calculate the current.

1.   Rearrange

I = V ÷ R

2.   Substitute

I = 12 ÷ 4.0

3.   Answer

I = 3.0 A

Answer:   3.0 A

Calculating Potential Difference

A current of 0.50 A flows through a 20 Ω resistor. Calculate the potential difference across it.

1.   Write the equation

V = IR

2.   Substitute

V = 0.50 × 20

3.   Answer

V = 10 V

Answer:   10 V

Calculating Resistance

A voltmeter reads 6.0 V and an ammeter reads 20 mA. Calculate the resistance.

1.   Convert the current

20 mA = 0.020 A

2.   Rearrange

R = V ÷ I

3.   Substitute

R = 6.0 ÷ 0.020

4.   Answer

R = 300 Ω

Answer:   300 Ω

Getting Good Results

Why we switch off between readings.

▸   Heating.   A current makes the wire hotter, and a hotter metal wire has a higher resistance.

▸   Constant temperature.   Use a low current and switch off between readings.

▸   Zero errors.   Check the meters read zero before starting.

▸   Repeat.   Repeat and average to reduce random errors.

Key Terms

| Resistance How much a component opposes the flow of current. | Ohm The unit of resistance (Ω). |
| --- | --- |
| Potential difference The energy transferred per coulomb; measured in volts. | Voltmeter Measures pd; connected in parallel. |
| Constant temperature Kept the same so the resistance does not change for other reasons. | Directly proportional Doubling one quantity doubles the other. |

Your Task: Predict the Graph

10 minutes

A student measures the resistance of 20 cm, 40 cm, 60 cm, 80 cm and 100 cm of a wire. What does the graph of resistance against length look like? A 30 cm length has a resistance of 1.8 Ω; what is the resistance of 90 cm? 1. Sketch the axes. 2. Use the ratio of lengths.

A good answer shows:   A straight line through the origin: resistance is directly proportional to length. 90 cm is three times as long, so R = 5.4 Ω.

Can I...?

☐   Recall V = IR.

☐   Rearrange for I or R.

☐   Convert mA to A.

☐   Describe the resistance of wire method.

☐   Explain why the wire is switched off between readings.

☐   Draw a graph of R against length.

☐   Say what happens to resistance in series and parallel.

☐   Use the correct units.

Summary

✓   V = IR.

✓   Ammeter in series, voltmeter in parallel.

✓   Longer wire means greater resistance.

✓   Switch off between readings to keep the temperature constant.

EXAM FOCUS A potential difference of 9.0 V is applied across a resistor and the current is 0.30 A. Calculate the resistance. (2 marks) State the equation, substitute and give the unit Ω.