AQA GCSE PHYSICS  ·  PAPER 1  ·  FOUNDATION & HIGHER

Resistors and I–V characteristics

Electricity   ·   Lesson 3 of 10

Warm-up

Answer each one, then check.

1.   What is an ohmic conductor?

A conductor with constant resistance at constant temperature

2.   What does I–V stand for?

Current against potential difference

3.   What is a straight line graph through the origin?

Directly proportional

4.   What happens to a metal when its temperature rises?

Its resistance rises

5.   What does LDR stand for?

Light-dependent resistor

Learning Objectives

1.   Describe the I–V graphs of a resistor, filament lamp and diode.

2.   Explain the changes in resistance of a filament lamp, diode, thermistor and LDR.

3.   Describe how to investigate I–V characteristics (Required Practical 4).

4.   Use graphs to say whether a component is linear or non-linear.

I–V Characteristics

The current through an ohmic conductor at constant temperature is directly proportional to the potential difference across it, so its resistance is constant.

The resistance of a filament lamp, diode, thermistor and LDR is not constant.

I–V Graphs

Straight line: constant resistance. Curve: changing resistance.

I-V graphs for a fixed resistor, a filament lamp and a diode.

Thermistor and LDR

Both fall as the quantity increases.

Graphs showing the resistance of a thermistor falling as temperature rises and the resistance of an LDR falling as light intensity rises.

Required Practical 4: I–V Circuit

Vary the pd and measure the current each time.

A circuit with a variable resistor, ammeter in series with the test component and a voltmeter across it.

Component Behaviour

Learn each description.

| Component | Resistance | Graph or behaviour |
| --- | --- | --- |
| Fixed resistor | Constant at constant temperature | Straight line through the origin |
| Filament lamp | Increases as the filament gets hotter | Curve that flattens; gradient decreases |
| Diode | Very high in the reverse direction | Current flows in one direction only |
| Thermistor | Decreases as temperature increases | Used in thermostats and temperature sensors |
| LDR | Decreases as light intensity increases | Used in automatic lights |

Required Practical 4: Method

Find the I–V characteristic of a component.

| 1 Set up Connect the component in series with the ammeter and variable resistor, and the voltmeter across the component. | 2 Vary Change the variable resistor to get different pds. | 3 Record For each setting record V and I. | 4 Reverse Swap the connections and repeat to get negative values (not for a diode with a protective resistor). | 5 Plot Draw current against pd and draw a smooth line. | 6 Repeat For the filament lamp and the diode. |
| --- | --- | --- | --- | --- | --- |

Resistance of a Filament Lamp

The current through a filament lamp is 0.34 A at 2.0 V and 0.62 A at 6.0 V. Calculate the resistance at each pd and explain the difference.

1.   At 2.0 V

R = 2.0 ÷ 0.34 = 5.9 Ω

2.   At 6.0 V

R = 6.0 ÷ 0.62 = 9.7 Ω

3.   Explain

At the higher pd the current is greater, so the filament is hotter and its resistance is higher

Answer:   5.9 Ω and 9.7 Ω; the resistance increases as the filament gets hotter.

Uses in Circuits

Applications you need to know.

▸   Thermistor.   In a thermostat: as the temperature rises the resistance falls, changing the pd across it to switch heating off.

▸   LDR.   In automatic lights: as it gets dark the resistance rises, which switches the light on.

▸   Diode.   Allows current in one direction only, protecting circuits from being connected the wrong way round.

Key Terms

| Ohmic conductor A conductor whose resistance is constant at constant temperature. | Filament lamp A lamp with a thin wire that gets hot and glows. |
| --- | --- |
| Diode A component that lets current flow in one direction only. | Thermistor A resistor whose resistance decreases when the temperature increases. |
| LDR A resistor whose resistance decreases when the light intensity increases. | Linear Following a straight line graph. |

Your Task: Match the Graph

10 minutes

Match each component to its I–V graph description: resistor, filament lamp, diode. (1) Curve that flattens as pd increases. (2) Straight line through the origin. (3) Almost no current one way, then a rapid increase the other way. 1. Look at the shape. 2. Explain the resistance change.

A good answer shows:   Resistor: (2). Filament lamp: (1). Diode: (3).

Can I...?

☐   Draw and describe the I–V graph of a resistor.

☐   Describe the I–V graph of a filament lamp.

☐   Describe the I–V graph of a diode.

☐   Explain why a filament lamp's resistance increases.

☐   Describe how a thermistor behaves.

☐   Describe how an LDR behaves.

☐   Describe Required Practical 4.

☐   Give one use each of a thermistor and an LDR.

Summary

✓   Resistor: constant resistance, straight line through the origin.

✓   Filament lamp: resistance increases with temperature.

✓   Diode: current in one direction only.

✓   Thermistor and LDR resistances decrease as temperature or light increases.

EXAM FOCUS Explain why the resistance of a filament lamp increases as the current increases. (3 marks) Say the filament gets hotter and link it to resistance.